Department of Electrical and Computer Engineering

COEN 6501                                                                               Dec. 9, 2016

Answer all Questions.                                All Questions carry equal marks

Exam Duration 3 hours

No books or  papers are allowed.                         Lecturer: Asim J. Al-Khalili

===============================================================

- Question  1

a. List programming techniques used in FPGAs and describe briefly two of these techniques.

b. A 2-bit by 2-bit multiplier of unsigned numbers A = a1 a0  and B = b1 b0 is to be implemented in  FPGA using Look Up Tables. Give the Tables contents and methods of accessing of the contents.

Determine total area and access time.  Assume the following:

Each memory cell has a delay of 0.5D and area of 2A

Each 2 to 1 MUX has a delay of D and area of A.

- Question 2

- Design the fastest circuit to implement the following function:

- F = (N-1)2         N is a 3-bit unsigned number

Evaluate your design in terms of speed and area. All gates have equal delays of 2ns and area Agate.

Question 3

Two unsigned binary numbers are arriving serially on lines A and B as shown in Fig.1.

It is required to add the numbers and produce the Sum (A plus B) at the output line. Design the sequential circuit starting with a state diagram.

Fig. 1  of Question 3

An………………A1 Ao

Sn+1……..S1 So

Bn ………………B1 Bo

Circuit to be

Designed

Question 4

a. Determine maximum speed of operation at typical conditions for the circuit shown in Fig. 2 below, taking into consideration the fan-out loading only. Timing parameters for all components are listed in Table 1.

b. The circuit is implemented on a die which is packaged in a ceramic DIP with a thermal resistance of 30oC/W. Calculate the drop in maximum speed of operation if the die dissipates a power of 2Watts at an ambient temperature of 40oC.

- Table 1 of Question 4

| - Component | - Tp (ns) | - Input - Loading (UL) | - K1 ns/UL | - K2 - ns/fo |
| --- | --- | --- | --- | --- |
| - Inverter | - 0.15 | - 1 | - 0.05 | - 0.1 |
| - NAND/NOR(2input) | - 0.25 | - 1.5 | - 0.1 | - 0.15 |
| - NAND (3 input) | - 0.5 | - 2 | - 0.15 | - 0.2 |
| - Flip Flop, ↑, (CK to Q) Tsu=1 ns, th = 0.5ns | - 1.5 | - 2.5 | - 0.2 | - 0.25 |

Fig. 2 of Question 4

Question 5

The System shown in the Fig. 3 and Table 2, below is running at 200 MHz.     a)Determine arrival time at point A.

b) Determine the set-up and the hold slack at Do andD1.

c) What is the maximum frequency that this system can run?

A

Fig. 3  and  Table 2

| - Component | - Tp (ns) | - Input - Loading (UL) | - K1 (ns/UL) | - K2 - (ns/fo) |
| --- | --- | --- | --- | --- |
| - XOR | - 0.3 | - 2 | - 0.2 | - 0.2 |
| - AND | - 0.2 | - 1.5 | - 0.15 | - 0.1 |
| D-Flip Flop tsu = 0.5ns, th = 0.2 ns | - 0.7 | - 3 | - 0.10 | - 0.3 |

Question 6

For the Multiplexer circuit shown below,

- Write a structural VHDL code for the entity MUX .

- Write VHDL code, Test Bench for testing the circuit

Test Bench

2 to 1 MUX

Generator

D1

C                                                        Z

D2

==============================================================================================

Td =(Tp+K1Ni+K2ML)∗K' ,       K’ = KT * KV *K  p

KT =    (Tj/Ta)  1.5                         TJ = Tamb + Φ Ja  * Pd

KV = 11+0.01∗fV    ,                           KP = 1+ 0.01 * fP