Concordia University, Department of Electrical and Computer Engineering DIGITAL DESIGN, COEN 212 Lecturers: Ted Obuchowicz , Asim Al-Khalili

Answer All Questions. Time Allowed 1 hr. 10min. Midterm Oct 12, 2017

Question 1

- Convert 11210  to Binary show your steps and give the results in Binary and Hexadecimal. (2 marks)

- Perform  the following operation using 6 bits 2’s complement arithmatic:

A B    where A= 1210   and  B= 1810  (3  Marks)

Question 2

(Use Boolean Algebra for all parts of Question 2)

2.a  Simplify to obtain minimum SOP   (3 marks)

F(a,b,c,d) = (a’b+ac+bc) + ab(a’c’d +cb’d’)

2.b  Minimize the following circuit, draw final minimized circuit. Determine delay before and after minimization . Numbers in the gates are in Nano seconds. AND gates and inverters have 10ns delays each.  (4 marks)

a

c

F

d

10

a

b

20

15

15

Question 3

3.a  Give minimal SOP for F(a,b,c,d) given by the following K-map

Identify all prime Implicants clearly. (3 marks)

cd

ab

00

01

11

10

00     01     11    10

1      X       X      1

1       X

1               1        X

X      X       X      X

3.b   Give the POS of F(A,B,C,D) =[ (A + ( C.D)] B   (2 mark)

3.c   Give the minimal NOR-NOR implementation of

F(A,B,C) = BC + A( B+C)       (2 marks)

## Question 4

# Draw timing diagram for the following input sequence 1001.

# X is initially set to 0 at  --            (6 marks)

X

F

Initial       t=0     10 ns            10 ns          10 ns             10 ns

X